Debounce Test Sheet / Test Log

Webinar 2 — GPIO / Interrupt / Button Bouncing

Environment

| Параметр | Значення |
| --- | --- |
| Board | ESP32 DevKit v1 |
| Input | BUTTON / NOISE → GPIO18 |
| Output | LED → GPIO2 |
| Simulator | Wokwi |
| Serial | 115200 |
| Build | GOOD / BUGGY: BUGGY |

Acceptance Criteria

Valid physical presses = logical press events. RAW interrupts may be greater. Short unstable noise must not create a false logical press/action.

Execution Log

| ID | Scenario | Physical presses | RAW | Logical | Actions | Expected | Actual | P/F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TC-01 | Normal press | 1 | 51 | 1 | 1 | 1 | 1 | P |
| TC-02 | Natural/Short bounce | 1 | 176 | 1 | 1 | 1 | 1 | P |
| TC-03 | Heavy bounce | 1 | 183 | 1 | 1 | 1 | 1 | P |
| TC-04 | Long press | 1 | 78 | 1 | 1 | 1 | 1 | P |
| TC-05 | Rapid presses | 5 | 527 | 5 | 5 | 5 | 5 | P |
| TC-06 | Short noise pulse | 0 | 2 | 1 | 0 | 0 | 1 | F |
| TC-07 | Noise during critical operation | 1 | 162 | 1 | 1 | 1 | 2 | F |

Logger Evidence Table

| Scenario | Physical actions | RAW interrupts / D0 | Logical events / D1 | Actions | Result |
| --- | --- | --- | --- | --- | --- |
| Normal press | 1 | 51 | 1 | 1 | PASS |
| Bounce | 1 | 176 | 1 | 1 | PASS |
| Long press | 1 | 78 | 1 | 1 | PASS |
| 5 rapid presses | 5 | 527 | 5 | 5 | PASS |
| Noise pulse | 0 | 2 | 1 | 0 | FAIL |
| Noise during critical operation | 1 | 162 | 1 | 1 | FAIL |

Bug Report for TC-06

Title: Помилкове спрацьовування системи при подачі короткого імпульсного шуму

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: 1. Система запущена у симуляторі Wokwi; 2. Кнопка у симуляторі не натиснута

Steps to Reproduce: 1. Подати короткий імпульсний сигнал шуму на  GPIO18; 2. Перевірити наявність виконаних дій (Action) і відсутність логічних подій (Logical events) у логах

Expected Result: Шум ігнорується системою, Actions = 0, Logical events (Expected) = 0

Actual Result: Виконалася логічна подія, Actions = 1, Logical events (Actual) = 1

Evidence:

System LOG:

================================

LOGICAL EVENT #1

Button press ACCEPTED

RAW interrupts total: 1

RAW interrupts since previous logical event: 1

LOGICAL events total: 1

ACTIONS total: 1

ACTION executed ONCE

LOGGER: physical press -> RAW -> LOGICAL -> ACTION

================================

[SYSTEM] Critical operation is running...

Button RELEASE accepted

[SERIAL] Wokwi UART alive

[LOGGER] RAW interrupts: 2 | Logical events: 1 | Actions: 1

[SYSTEM] Critical operation finished.

Reproducibility: 100% (відтворено 5 з 5 разів)

Severity: High

Bug Report for TC-07

Title: Помилкове спрацьовування системи при подачі короткого імпульсного шуму під час виконання критичної операції

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: 1. Система запущена у симуляторі Wokwi; 2. Система знаходиться в стані виконання критичної операції

Steps to Reproduce: 1. Натиснути кнопку на GPIO18; 2. Під час виконання критичної операції подати на GPIO18 короткий імпульсний сигнал шуму; 3. Перевірити підсумковий стан дій (поява шуму під час виконання критичної операції не повинно викликати додаткову логічну подію)

Expected Result: Поява шуму не впливає на результат виконання фізичної дії, Actions = 1, Logical events (Expected) = 1

Actual Result: Виконалася додаткова фізична дія і логічна подія, Actions = 2, Logical events (Actual) = 2

Evidence:

System LOG:

ESP32 BUTTON / INTERRUPT / DEBOUNCE DEMO

----------------------------------------

GPIO18 -> INTERRUPT -> ISR -> DEBOUNCE -> EVENT

Green BUTTON = physical button with bouncing

Red NOISE    = artificial noise input

[SERIAL] Wokwi UART alive

[SERIAL] Wokwi UART alive

================================

LOGICAL EVENT #1

Button press ACCEPTED

RAW interrupts total: 5

RAW interrupts since previous logical event: 5

LOGICAL events total: 1

ACTIONS total: 1

ACTION executed ONCE

LOGGER: physical press -> RAW -> LOGICAL -> ACTION

================================

[SYSTEM] Critical operation is running...

Button RELEASE accepted

================================

LOGICAL EVENT #2

Button press ACCEPTED

RAW interrupts total: 110

RAW interrupts since previous logical event: 105

LOGICAL events total: 2

ACTIONS total: 2

ACTION executed ONCE

LOGGER: physical press -> RAW -> LOGICAL -> ACTION

================================

Button RELEASE accepted

[SERIAL] Wokwi UART alive

[LOGGER] RAW interrupts: 111 | Logical events: 2 | Actions: 2

[SYSTEM] Critical operation finished.

[SERIAL] Wokwi UART alive

[SERIAL] Wokwi UART alive

Reproducibility: 100% (відтворено 5 з 5 разів)

Severity: High

QA Conclusion

У межах Embedded QA-тестування обробки переривань GPIO18 було оцінено стійкість прошивки (Build: BUGGY) до брязкоту контактів, електричних перешкод і високої інтенсивності вхідних сигналів. За результатами тестування встановлено, що основна вимога acceptance criteria не виконується: кількість зареєстрованих фізичних натискань (дій) не завжди відповідає фактичній кількості логічних подій. Під час TC-06 було виявлено повну відсутність захисту сигнального алгоритму від короткочасних шумових імпульсів, унаслідок чого можливі хибні спрацювання та несанкціоноване виконання дій. У TC-07, під час виконання критичної операції за умов наявності шуму, система некоректно зафіксувала логічну подію.

Основними виявленими ризиками є перевантаження ISR через велику кількість переривань та хибні спрацювання внаслідок електричних наводок.

Для усіх кейсів RAW interrupts (D0) більші за кількість фізичних натискань. В основному це не призвело до зайвої logical event/action (D1), тільки у TC-06 шум перетворився на зайву логічну подію.