Recovery / Fault Injection Test Sheet

Webinar 5 — Sleep, WDT, Power Loss, Persistence

# Environment

| Field | Value |
| --- | --- |
| MCU | ESP32 DevKit V1 |
| Sensor | DHT22 GPIO15 |
| Button | GPIO4 |
| LED | GPIO2 |
| Relay | GPIO5 |
| Serial | 115200 |
| Build | DEMO / BUGGY: BUGGY |

# Test Matrix

| Scenario | Risk | Expected | Actual | Evidence | P/F |
| --- | --- | --- | --- | --- | --- |
| Sleep 5 | Timing / wake / persistence | За вимогами пристрій переходить у deep sleep на 5 с., після wake-up пристрій працездатний, persistent data залишаються цілісними | Пристрій переходить у deep sleep, нема стану wake-up, натомість пристрій перезавантажується, перед переходом у deep sleep дані зберігаються | rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true} [STORE] saved [STATE] SLEEP timer=5 s ets Jul 29 2019 12:21:46 rst:0x5 (DEEPSLEEP_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=DEEP_SLEEP [WAKE] cause=4 [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"DEEP_SLEEP","relay":false,"counter":1,"heap":336332,"sensor_ok":true} | F |
| WDT hang | Reset time / reason / recovery | За вимогами WDT перезавантажує пристрій приблизно не пізніше ніж за 5 секунд, reset reason – WDT reset, після reset пристрій працездатний, actuator після аварійного reset має бути OFF, boot loop відсутній | WDT перезавантажив пристрій у вказаний час не пізніше ніж за 5 с., reset reason вірний – WDT reset, після reset пристрій працездатний, boot loop відсутній. Підтвердження роботи WDT у файлі WDT.gif | rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true} [FAULT] firmware hang E (22096) task_wdt: Task watchdog got triggered. The following tasks did not reset the watchdog in time: E (22096) task_wdt: - loopTask (CPU 1) E (22096) task_wdt: Tasks currently running: E (22096) task_wdt: CPU 0: IDLE0 E (22096) task_wdt: CPU 1: loopTask E (22096) task_wdt: Aborting. abort() was called at PC 0x400dbf11 on core 0 Backtrace: 0x400835b5:0x3ffbea8c ¦<-CORRUPTED ELF file SHA256: 389836db4aa63421 ets Jul 29 2019 12:21:46 rst:0x8 (TG1WDT_SYS_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=INT_WDT [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"INT_WDT","relay":false,"counter":0,"heap":336332,"sensor_ok":true} | P |
| Relay + WDT | Safe actuator state | За вимогами WDT перезавантажує пристрій приблизно не пізніше ніж за 5 секунд, reset reason – WDT reset, після reset пристрій працездатний, actuator після аварійного reset має бути OFF, boot loop відсутній | WDT перезавантажив пристрій у вказаний час не пізніше ніж за 5 с., reset reason вірний – WDT reset, після reset пристрій працездатний, boot loop відсутній АЛЕ actuator після аварійного reset ON, тобто relay має статус true | rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true} [STORE] saved [RELAY] ON [FAULT] firmware hang E (24664) task_wdt: Task watchdog got triggered. The following tasks did not reset the watchdog in time: E (24664) task_wdt: - loopTask (CPU 1) E (24664) task_wdt: Tasks currently running: E (24664) task_wdt: CPU 0: IDLE0 E (24664) task_wdt: CPU 1: loopTask E (24664) task_wdt: Aborting. abort() was called at PC 0x400dbf11 on core 0 Backtrace: 0x400835b5:0x3ffbea8c ¦<-CORRUPTED ELF file SHA256: 389836db4aa63421 ets Jul 29 2019 12:21:46 rst:0x8 (TG1WDT_SYS_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=INT_WDT [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"INT_WDT","relay":true,"counter":1,"heap":336332,"sensor_ok":true} | F |
| Interrupted save | Atomicity / integrity | За вимогами після reboot має залишитися стара повністю валідна або нова повністю валідна версія стану, змішаний/частково оновлений state не допускається, corrupted state має виявлятися | Після reboot залишається невалідна версія стану — до reboot було "counter":1, після reboot – "counter":101, тобто маємо змішаний/частково оновлений state, цей змішаний state не детектується | rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true} [STORE] saved {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":1,"heap":336332,"sensor_ok":true} [FAULT] power loss DURING save ets Jul 29 2019 12:21:46 rst:0xc (SW_CPU_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=SOFTWARE_RESET [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"SOFTWARE_RESET","relay":false,"counter":101,"heap":336332,"sensor_ok":true} | F |
| Sensor failure | Explicit invalid-data handling | За вимогами firmware не повинна трактувати це як валідне вимірювання, у логах має бути явна помилка, actuator не повинен переходити у небезпечний стан через invalid data | Команда status показує "sensor_ok":false, але інша команда sensor виводить валідні значення - [SENSOR] temp=0.0C humidity=0.0%, хоча має виводити помилку, тому що сенсор фізично не підключено - {"status":"ERROR","code":"SENSOR_UNAVAILABLE"} | ets Jul 29 2019 12:21:46 rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":false} [SENSOR] temp=0.0C humidity=0.0% | F |
| Button | Single logical event | За вимогами один фізичний натиск кнопки повинен створити не більше однієї логічної події. | Після декількох натискань кнопки з’являється 2 логічні події | rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true} [STORE] saved [BUTTON] relay=ON {"state":"ACTIVE","reset_reason":"POWER_ON","relay":true,"counter":1,"heap":336332,"sensor_ok":true} [STORE] saved [BUTTON] relay=OFF {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":2,"heap":336332,"sensor_ok":true} [STORE] saved [BUTTON] relay=ON {"state":"ACTIVE","reset_reason":"POWER_ON","relay":true,"counter":3,"heap":336332,"sensor_ok":true} [STORE] saved [BUTTON] relay=OFF [STORE] saved [BUTTON] relay=ON {"state":"ACTIVE","reset_reason":"POWER_ON","relay":true,"counter":5,"heap":336332,"sensor_ok":true} | F |
| Repeated recovery | No boot loop / stable | За вимогами після декількох зависань і перезавантажень пристрій не має переходити у режим boot loop та перезавантаження має бути стабільним | Після декількох зависань і перезавантажень пристрій не переходить у режим boot loop та перезавантаження стабільно повертає пристрій у робочий режим | rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=POWER_ON [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot [FAULT] firmware hang E (32670) task_wdt: Task watchdog got triggered. The following tasks did not reset the watchdog in time: E (32670) task_wdt: - loopTask (CPU 1) E (32670) task_wdt: Tasks currently running: E (32670) task_wdt: CPU 0: IDLE0 E (32670) task_wdt: CPU 1: loopTask E (32670) task_wdt: Aborting. abort() was called at PC 0x400dbf11 on core 0 Backtrace: 0x400835b5:0x3ffbea8c ¦<-CORRUPTED ELF file SHA256: 3f75e9f371502bbc ets Jul 29 2019 12:21:46 rst:0x8 (TG1WDT_SYS_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=INT_WDT [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot [FAULT] firmware hang E (24531) task_wdt: Task watchdog got triggered. The following tasks did not reset the watchdog in time: E (24531) task_wdt: - loopTask (CPU 1) E (24531) task_wdt: Tasks currently running: E (24531) task_wdt: CPU 0: IDLE0 E (24531) task_wdt: CPU 1: loopTask E (24531) task_wdt: Aborting. abort() was called at PC 0x400dbf11 on core 0 Backtrace: 0x400835b5:0x3ffbea8c ¦<-CORRUPTED ELF file SHA256: 3f75e9f371502bbc ets Jul 29 2019 12:21:46 rst:0x8 (TG1WDT_SYS_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=INT_WDT [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot [FAULT] firmware hang E (14307) task_wdt: Task watchdog got triggered. The following tasks did not reset the watchdog in time: E (14307) task_wdt: - loopTask (CPU 1) E (14307) task_wdt: Tasks currently running: E (14307) task_wdt: CPU 0: IDLE0 E (14307) task_wdt: CPU 1: loopTask E (14307) task_wdt: Aborting. abort() was called at PC 0x400dbf11 on core 0 Backtrace: 0x400835b5:0x3ffbea8c ¦<-CORRUPTED ELF file SHA256: 3f75e9f371502bbc ets Jul 29 2019 12:21:46 rst:0x8 (TG1WDT_SYS_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT) configsip: 0, SPIWP:0xee clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00 mode:DIO, clock div:2 load:0x3fff0030,len:1156 load:0x40078000,len:11456 ho 0 tail 12 room 4 load:0x40080400,len:2972 entry 0x400805dc [BOOT] reset_reason=INT_WDT [WDT] enabled timeout=4000 ms Commands: help status sensor relay on relay off save sleep <seconds> hang powerloss reboot | P |

# Bug Report Sleep_5

Title: Команда sleep 5 переводить пристрій у сон на 0,5 секунд замість 5 секунд

Requirement: похибка wake-up ±15%; після wake-up пристрій працездатний; persistent data залишаються цілісними.

Risk: Timing / wake / persistence

Preconditions: симуляція запущена

система має наступний статус {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

Steps: ввести команду sleep 5

Expected: пристрій переходить у deep sleep на 5 с., після wake-up пристрій працездатний, persistent data залишаються цілісними

Actual: пристрій переходить у deep sleep,але пробуджується надто швидко, замість 5 секунд сон триває 0,5 секунд, перед переходом у deep sleep дані зберігаються. Файл доказу швидкого виходу із сну - Sleep_5.gif

Timing / Reset Reason: "reset_reason":"DEEP_SLEEP"

Evidence:

rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=POWER_ON

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

[STORE] saved

[STATE] SLEEP timer=5 s

ets Jul 29 2019 12:21:46

rst:0x5 (DEEPSLEEP_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=DEEP_SLEEP

[WAKE] cause=4

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"DEEP_SLEEP","relay":false,"counter":1,"heap":336332,"sensor_ok":true}

Severity: Critical

Reproducibility: 100% (відтворено 3 з 3 разів)

# Bug Report Relay_+_WDT

Title: Після відпрацювання WDT і перезавантаження пристрою relay після аварійного reset має статус ON

Requirement: WDT перезавантажує пристрій приблизно не пізніше ніж за 5 секунд; reset reason дозволяє визначити WDT reset; після reset пристрій працездатний; actuator після аварійного reset має бути OFF; boot loop відсутній

Risk: Safe actuator state

Preconditions: симуляція запущена

система має наступний статус {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

Steps: ввести команду relay on, наступною ввести команду hang

Expected: WDT перезавантажує пристрій приблизно не пізніше ніж за 5 секунд, reset reason – WDT reset, після reset пристрій працездатний, actuator після аварійного reset має бути OFF,  boot loop відсутній

Actual: після перезавантаження пристрою actuator після аварійного reset має статус ON, тобто має бути "relay":false, а не "relay": true

Timing / Reset Reason: "reset_reason":"INT_WDT"

Evidence:

rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=POWER_ON

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

[STORE] saved

[RELAY] ON

[FAULT] firmware hang

E (24664) task_wdt: Task watchdog got triggered. The following tasks did not reset the watchdog in time:

E (24664) task_wdt:  - loopTask (CPU 1)

E (24664) task_wdt: Tasks currently running:

E (24664) task_wdt: CPU 0: IDLE0

E (24664) task_wdt: CPU 1: loopTask

E (24664) task_wdt: Aborting.

abort() was called at PC 0x400dbf11 on core 0

Backtrace: 0x400835b5:0x3ffbea8c |<-CORRUPTED

ELF file SHA256: 389836db4aa63421

ets Jul 29 2019 12:21:46

rst:0x8 (TG1WDT_SYS_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=INT_WDT

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"INT_WDT","relay":true,"counter":1,"heap":336332,"sensor_ok":true}

Severity: Critical

Reproducibility: 100% (відтворено 3 з 3 разів)

# Bug Report Sensor_failure

Title: Команда sensor при фізично відключеному сенсорі повертає валідне значення вимірів

Requirement: Якщо DHT22 недоступний або повертає некоректні дані: firmware не повинна трактувати це як валідне вимірювання; у логах має бути явна помилка; actuator не повинен переходити у небезпечний стан через invalid data.

Risk: Explicit invalid-data handling

Preconditions: симуляція запущена

з’єднання датчика із МК було змінено, у симуляції датчик під’єднано до GPIO15, змінено на GPIO13

система має наступний статус {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":false}

Steps: ввести команду sensor

Expected: у логах має бути помилка {"status":"ERROR","code":"SENSOR_UNAVAILABLE"}

Actual: у логах з’являється валідне значення [SENSOR] temp=0.0C humidity=0.0%

Timing / Reset Reason: "reset_reason":"POWER_ON"

Evidence:

ets Jul 29 2019 12:21:46

rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=POWER_ON

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":false}

[SENSOR] temp=0.0C humidity=0.0%

Severity: High

Reproducibility: 100% (відтворено 3 з 3 разів)

# Bug Report Interrupted_save

Title: Після перезавантаження пристрою значення параметра counter не скидається на 0

Requirement: після reboot має залишитися стара повністю валідна або нова повністю валідна версія стану; змішаний/частково оновлений state не допускається; corrupted state має виявлятися.

Risk: Atomicity / integrity

Preconditions: симуляція запущена

система має наступний статус {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

Steps: ввести команду status, ввести команду save,  ввести команду status,  ввести команду powerloss,  ввести команду status

Expected: після reboot має залишитися стара повністю валідна або нова повністю валідна версія стану, змішаний/частково оновлений state не допускається, corrupted state має виявлятися

Actual: після reboot залишається невалідна версія стану — до reboot було "counter":1, після reboot – "counter":101, тобто маємо змішаний/частково оновлений state, цей змішаний state не детектується

Timing / Reset Reason: "reset_reason":"SOFTWARE_RESET"

Evidence:

rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=POWER_ON

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

[STORE] saved

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":1,"heap":336332,"sensor_ok":true}

[FAULT] power loss DURING save

ets Jul 29 2019 12:21:46

rst:0xc (SW_CPU_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=SOFTWARE_RESET

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"SOFTWARE_RESET","relay":false,"counter":101,"heap":336332,"sensor_ok":true}

Severity: High

Reproducibility: 100% (відтворено 3 з 3 разів)

# Bug Report Button

Title: Після натискання кнопки з’являється дві логічні події

Requirement: Один фізичний натиск кнопки повинен створити не більше однієї логічної події.

Risk: Single logical event

Preconditions: симуляція запущена

система має наступний статус {"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

Steps: настиснути кнопку у симуляції, ввести команду status

Expected: одна фізична дія повинна створювати одну логічну подію

Actual: на четвертому натисканні кнопки з’являється декілька логічних подій, що відображає команда status після натискання, тобто не спрацьовує debounce кнопки

Timing / Reset Reason: "reset_reason":"POWER_ON"

Evidence:

rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

[BOOT] reset_reason=POWER_ON

[WDT] enabled timeout=4000 ms

Commands:

help

status

sensor

relay on

relay off

save

sleep <seconds>

hang

powerloss

reboot

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":0,"heap":336332,"sensor_ok":true}

[STORE] saved

[BUTTON] relay=ON

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":true,"counter":1,"heap":336332,"sensor_ok":true}

[STORE] saved

[BUTTON] relay=OFF

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":false,"counter":2,"heap":336332,"sensor_ok":true}

[STORE] saved

[BUTTON] relay=ON

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":true,"counter":3,"heap":336332,"sensor_ok":true}

[STORE] saved

[BUTTON] relay=OFF

[STORE] saved

[BUTTON] relay=ON

{"state":"ACTIVE","reset_reason":"POWER_ON","relay":true,"counter":5,"heap":336332,"sensor_ok":true}

Severity: Major

Reproducibility: 100% (відтворено 3 з 3 разів)

# Release Decision

| Decision | GO / CONDITIONAL GO / NO-GO: NO-GO |
| --- | --- |
| Blocking risks | Atomicity / integrity, Safe actuator state, Timing / wake / persistence, Single logical event |
| Tests required on real hardware | Так, тому що brown-out та електричні характеристики живлення не можна перевірити на симуляції |

# QA Conclusion

Пристрій не готовий, тому що залишається дуже багато critical risks, серед яких Atomicity / integrity, Safe actuator state, Timing / wake / persistence. Deep sleep дуже швидко виводить пристрій із сну, після WDT reset не скидається, а безумовно відновлюється relay state, пристрій відповідає валідним значенням вимірів при фізично вимкненому сенсорі. Також на реальному пристрої треба перевірити brown-out та електричні характеристики живлення