Python/Pytest Automation Test Sheet

Webinar 6 — execution, triage та evidence

# Environment

| Field | Value |
| --- | --- |
| Python | 3.15.2 |
| pytest | 9.2.0 |
| pyserial | 3.5 |
| DEVICE_PORT | rfc2217://localhost:4000 |
| Firmware | BUGGY |
| Serial | 115200 |

# Execution Log

| ID | Test / Input | Expected | Actual | Classification | Evidence | P/F |
| --- | --- | --- | --- | --- | --- | --- |
| T-01 | test_ping Command PING | Test passed | TimeoutError: No response for command: PING | Requirement | Bug report T-01 | F |
| T-02 | test_status Command STATUS | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","led":false,"pwm":0,"heap":350140} {"status":"OK","reset":true} | P |
| T-03 | test_temperature_is_valid_json Command GET TEMP | Test passed | AssertionError: Invalid JSON from device: '{"status":"OK","temperature":24.5' | Firmware | Bug report T-03 | F |
| T-04 | test_led_on_off Command SET LED ON SET LED OFF | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","led":true} {"status":"OK","led":false} {"status":"OK","reset":true} | P |
| T-05 | test_reset_restores_safe_state Command SET LED ON RESET STATUS | Test passed | assert 255 == 0 | Firmware | Bug report T-05 | F |
| T-06 | test_unknown_command Command PONG | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"UNKNOWN_COMMAND"} {"status":"OK","reset":true} | P |
| T-07 | test_no_response_becomes_timeout Command NO_RESPONSE | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","reset":true} {"status":"OK","reset":true} | P |
| T-08 | test_pwm_valid_boundaries Command SET PWM 0 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":0} {"status":"OK","reset":true} | P |
| T-9 | test_pwm_valid_boundaries Command SET PWM 1 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":1} {"status":"OK","reset":true} | P |
| T-10 | test_pwm_valid_boundaries Command SET PWM 50 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":50} {"status":"OK","reset":true} | P |
| T-11 | test_pwm_valid_boundaries Command SET PWM 99 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":99} {"status":"OK","reset":true} | P |
| T-12 | test_pwm_valid_boundaries Command SET PWM 100 | Test passed | AssertionError: assert 'ERROR' == 'OK' | Firmware | Bug report T-12 | F |
| T-13 | test_pwm_invalid_boundaries Command SET PWM -1 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"INVALID_VALUE"} {"status":"OK","reset":true} | P |
| T-14 | test_pwm_invalid_boundaries Command SET PWM 101 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"INVALID_VALUE"} {"status":"OK","reset":true} | P |
| T-15 | test_pwm_invalid_boundaries Command SET PWM 999 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"INVALID_VALUE"} {"status":"OK","reset":true} | P |
| T-16 | test_pwm_additional_valid_values Command SET PWM 15 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":15} {"status":"OK","reset":true} | P |
| T-17 | test_pwm_additional_valid_values Command SET PWM 75 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":75} {"status":"OK","reset":true} | P |
| T-18 | test_pwm_additional_valid_values Command SET PWM 98 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"OK","pwm":98} {"status":"OK","reset":true} | P |
| T-19 | test_pwm_additional_invalid_values Command SET PWM -101 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"INVALID_VALUE"} {"status":"OK","reset":true} | P |
| T-20 | test_pwm_additional_invalid_values Command SET PWM 10.5 | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"INVALID_VALUE"} {"status":"OK","reset":true} | P |
| T-21 | test_pwm_additional_invalid_values Command SET PWM ‘’ | Test passed | Test passed |  | {"status":"OK","reset":true} {"status":"ERROR","code":"INVALID_VALUE"} {"status":"OK","reset":true} | P |

# FAIL Classification

| Category | Check | Evidence |
| --- | --- | --- |
| Test | Assert/oracle correct? | Test code, expected value |
| Environment | DEVICE_PORT/dependencies/port ownership? | Configuration, logs |
| Communication | Timeout/transport/raw bytes? | Serial logs, received bytes |
| Requirement | Expected explicitly supported? | Requirement ID |
| Firmware | Repeatable mismatch with requirement? | Actual response, repeated test results |

# Device LOG

ets Jul 29 2019 12:21:46

rst:0x1 (POWERON_RESET),boot:0x13 (SPI_FAST_FLASH_BOOT)

configsip: 0, SPIWP:0xee

clk_drv:0x00,q_drv:0x00,d_drv:0x00,cs0_drv:0x00,hd_drv:0x00,wp_drv:0x00

mode:DIO, clock div:2

load:0x3fff0030,len:1156

load:0x40078000,len:11456

ho 0 tail 12 room 4

load:0x40080400,len:2972

entry 0x400805dc

{"status":"READY"}

{"status":"OK","reset":true}

{"status":"OK","command":"PING"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","led":false,"pwm":0,"heap":350140}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","temperature":24.5

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","led":true}

{"status":"OK","led":false}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","led":true}

{"status":"OK","reset":true}

{"status":"OK","led":true,"pwm":255,"heap":349852}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"UNKNOWN_COMMAND"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":0}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":1}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":50}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":99}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":15}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":75}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"OK","pwm":98}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

# Pytest response

(.venv) PS C:\Users\ober-pc\Documents\git\EmbededQA\pytest\automation> python -m pytest -v .\tests\test_firmware_cli.py

==================================================================== test session starts =====================================================================

platform win32 -- Python 3.15.2, pytest-9.2.0, pluggy-1.7.0

rootdir: C:\Users\ober-pc\Documents\git\EmbededQA\pytest\automation

configfile: pytest.ini

collected 21 items

tests\test_firmware_cli.py F.F.F......F.........                                                                                                        [100%]

========================================================================== FAILURES ==========================================================================

_________________________________________________________________________ test_ping __________________________________________________________________________

device = <device.Device object at 0x000002AF4C724830>

def test_ping(device):

>       r = device.command("PING")

^^^^^^^^^^^^^^^^^^^^^^

tests\test_firmware_cli.py:4:

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

device.py:27: in command

raw = self.command_raw(command)

^^^^^^^^^^^^^^^^^^^^^^^^^

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

self = <device.Device object at 0x000002AF4C724830>, command = 'PING'

def command_raw(self, command):

self.ser.reset_input_buffer()

self.ser.write((command + "\n").encode())

self.ser.flush()

line = self.ser.readline().decode(errors="replace").strip()

if not line:

>           raise TimeoutError(f"No response for command: {command}")

E           TimeoutError: No response for command: PING

device.py:23: TimeoutError

_______________________________________________________________ test_temperature_is_valid_json _______________________________________________________________

self = <device.Device object at 0x000002AF4C7CE0D0>, command = 'GET TEMP'

def command(self, command):

raw = self.command_raw(command)

try:

>           return json.loads(raw)

^^^^^^^^^^^^^^^

device.py:29:

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

..\..\..\..\..\AppData\Local\Python\pythoncore-3.15-64\Lib\json\__init__.py:371: in loads

return _default_decoder.decode(s)

^^^^^^^^^^^^^^^^^^^^^^^^^^

..\..\..\..\..\AppData\Local\Python\pythoncore-3.15-64\Lib\json\decoder.py:358: in decode

obj, end = self.raw_decode(s, idx=_w(s, 0).end())

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

self = <json.decoder.JSONDecoder object at 0x000002AF49E0C050>, s = '{"status":"OK","temperature":24.5', idx = 0

def raw_decode(self, s, idx=0):

"""Decode a JSON document from ``s`` (a ``str`` beginning with

a JSON document) and return a 2-tuple of the Python

representation and the index in ``s`` where the document ended.

This can be used to decode a JSON document from a string that may

have extraneous data at the end.

"""

try:

>           obj, end = self.scan_once(s, idx)

^^^^^^^^^^^^^^^^^^^^^^

E           json.decoder.JSONDecodeError: Expecting ',' delimiter: line 1 column 34 (char 33)

..\..\..\..\..\AppData\Local\Python\pythoncore-3.15-64\Lib\json\decoder.py:374: JSONDecodeError

The above exception was the direct cause of the following exception:

device = <device.Device object at 0x000002AF4C7CE0D0>

def test_temperature_is_valid_json(device):

>       r = device.command("GET TEMP")

^^^^^^^^^^^^^^^^^^^^^^^^^^

tests\test_firmware_cli.py:15:

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

self = <device.Device object at 0x000002AF4C7CE0D0>, command = 'GET TEMP'

def command(self, command):

raw = self.command_raw(command)

try:

return json.loads(raw)

except json.JSONDecodeError as e:

>           raise AssertionError(f"Invalid JSON from device: {raw!r}") from e

E           AssertionError: Invalid JSON from device: '{"status":"OK","temperature":24.5'

device.py:31: AssertionError

_______________________________________________________________ test_reset_restores_safe_state _______________________________________________________________

device = <device.Device object at 0x000002AF4C80CD60>

def test_reset_restores_safe_state(device):

device.command("SET LED ON")

device.command("RESET")

status = device.command("STATUS")

>       assert status["pwm"] == 0

E       assert 255 == 0

tests\test_firmware_cli.py:28: AssertionError

_______________________________________________________________ test_pwm_valid_boundaries[100] _______________________________________________________________

device = <device.Device object at 0x000002AF4C71A990>, value = 100

@pytest.mark.parametrize("value", [0, 1, 50, 99, 100])

def test_pwm_valid_boundaries(device, value):

r = device.command(f"SET PWM {value}")

>       assert r["status"] == "OK"

E       AssertionError: assert 'ERROR' == 'OK'

E

E         - OK

E         + ERROR

tests\test_firmware_cli.py:43: AssertionError

====================================================================== warnings summary ======================================================================

tests/test_firmware_cli.py: 21 warnings

C:\Users\ober-pc\Documents\git\EmbededQA\pytest\automation\.venv\Lib\site-packages\serial\rfc2217.py:465: DeprecationWarning: setDaemon() is deprecated, set the daemon attribute instead

self._thread.setDaemon(True)

tests/test_firmware_cli.py: 21 warnings

C:\Users\ober-pc\Documents\git\EmbededQA\pytest\automation\.venv\Lib\site-packages\serial\rfc2217.py:466: DeprecationWarning: setName() is deprecated, set the name attribute instead

self._thread.setName('pySerial RFC 2217 reader thread for {}'.format(self._port))

-- Docs: https://docs.pytest.org/en/stable/how-to/capture-warnings.html

================================================================== short test summary info ===================================================================

FAILED tests/test_firmware_cli.py::test_ping - TimeoutError: No response for command: PING

FAILED tests/test_firmware_cli.py::test_temperature_is_valid_json - AssertionError: Invalid JSON from device: '{"status":"OK","temperature":24.5'

FAILED tests/test_firmware_cli.py::test_reset_restores_safe_state - assert 255 == 0

FAILED tests/test_firmware_cli.py::test_pwm_valid_boundaries[100] - AssertionError: assert 'ERROR' == 'OK'

========================================================= 4 failed, 17 passed, 42 warnings in 43.91s =========================================================

# Bug Report T-01

Title: Команда PING відповідає TimeoutError: No response for command: PING

Requirement: `PING` → valid JSON with `status=OK`

Environment: Python 3.15.2, pytest 9.2.0, pyserial 3.5, DEVICE_PORT rfc2217://localhost:4000, Firmware BUGGY, Serial 115200

Preconditions: система після команди RESET {"status":"OK","reset":true}

Test/Steps: запустити тест test_ping

Expected: test passed, {"status":"OK","command":"PING"}

Actual: TimeoutError: No response for command: PING

Raw Serial Evidence: {"status":"OK","command":"PING"}

Pytest Evidence:

_________________________________________________________________________ test_ping __________________________________________________________________________

device = <device.Device object at 0x0000023D9BCE4830>

def test_ping(device):

>       r = device.command("PING")

^^^^^^^^^^^^^^^^^^^^^^

tests\test_firmware_cli.py:4:

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

device.py:27: in command

raw = self.command_raw(command)

^^^^^^^^^^^^^^^^^^^^^^^^^

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

self = <device.Device object at 0x0000023D9BCE4830>, command = 'PING'

def command_raw(self, command):

self.ser.reset_input_buffer()

self.ser.write((command + "\n").encode())

self.ser.flush()

line = self.ser.readline().decode(errors="replace").strip()

if not line:

>           raise TimeoutError(f"No response for command: {command}")

E           TimeoutError: No response for command: PING

device.py:23: TimeoutError

Severity: Major

Reproducibility: 100% (відтворено 4 з 4 разів)

# Bug Report T-03

Title: Команда GET TEMP відповідає AssertionError: Invalid JSON from device: '{"status":"OK","temperature":24.5'

Requirement: `GET TEMP` → valid JSON; either valid temperature or explicit sensor error

Environment: Python 3.15.2, pytest 9.2.0, pyserial 3.5, DEVICE_PORT rfc2217://localhost:4000, Firmware BUGGY, Serial 115200

Preconditions: система після команди RESET {"status":"OK","reset":true}

Test/Steps: запустити тест test_temperature_is_valid_json

Expected: test passed, {"status":"OK","temperature":24.5}

Actual: AssertionError: Invalid JSON from device: '{"status":"OK","temperature":24.5'

Raw Serial Evidence:

{"status":"OK","reset":true}

{"status":"OK","temperature":24.5

{"status":"OK","reset":true}

Pytest Evidence:

_______________________________________________________________ test_temperature_is_valid_json _______________________________________________________________

self = <device.Device object at 0x0000023D9BDA20D0>, command = 'GET TEMP'

def command(self, command):

raw = self.command_raw(command)

try:

>           return json.loads(raw)

^^^^^^^^^^^^^^^

device.py:29:

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

..\..\..\..\..\AppData\Local\Python\pythoncore-3.15-64\Lib\json\__init__.py:371: in loads

return _default_decoder.decode(s)

^^^^^^^^^^^^^^^^^^^^^^^^^^

..\..\..\..\..\AppData\Local\Python\pythoncore-3.15-64\Lib\json\decoder.py:358: in decode

obj, end = self.raw_decode(s, idx=_w(s, 0).end())

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

self = <json.decoder.JSONDecoder object at 0x0000023D9937C050>, s = '{"status":"OK","temperature":24.5', idx = 0

def raw_decode(self, s, idx=0):

"""Decode a JSON document from ``s`` (a ``str`` beginning with

a JSON document) and return a 2-tuple of the Python

representation and the index in ``s`` where the document ended.

This can be used to decode a JSON document from a string that may

have extraneous data at the end.

"""

try:

>           obj, end = self.scan_once(s, idx)

^^^^^^^^^^^^^^^^^^^^^^

E           json.decoder.JSONDecodeError: Expecting ',' delimiter: line 1 column 34 (char 33)

..\..\..\..\..\AppData\Local\Python\pythoncore-3.15-64\Lib\json\decoder.py:374: JSONDecodeError

The above exception was the direct cause of the following exception:

device = <device.Device object at 0x0000023D9BDA20D0>

def test_temperature_is_valid_json(device):

>       r = device.command("GET TEMP")

^^^^^^^^^^^^^^^^^^^^^^^^^^

tests\test_firmware_cli.py:15:

_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _

self = <device.Device object at 0x0000023D9BDA20D0>, command = 'GET TEMP'

def command(self, command):

raw = self.command_raw(command)

try:

return json.loads(raw)

except json.JSONDecodeError as e:

>           raise AssertionError(f"Invalid JSON from device: {raw!r}") from e

E           AssertionError: Invalid JSON from device: '{"status":"OK","temperature":24.5'

device.py:31: AssertionError

Severity: High

Reproducibility: 100% (відтворено 4 з 4 разів)

# Bug Report T-05

Title: Команда RESET не скинула значення параметрів led та pwm у safe state

Requirement: `RESET` restores safe output state: LED/PWM OFF/0

Environment: Python 3.15.2, pytest 9.2.0, pyserial 3.5, DEVICE_PORT rfc2217://localhost:4000, Firmware BUGGY, Serial 115200

Preconditions: система після команди RESET {"status":"OK","reset":true}

Test/Steps: запустити тест test_reset_restores_safe_state

Expected: test passed, {"status":"OK","led":false,"pwm":0,"heap":349852}

Actual: assert 255 == 0

Raw Serial Evidence:

{"status":"OK","reset":true}

{"status":"OK","led":true}

{"status":"OK","reset":true}

{"status":"OK","led":true,"pwm":255,"heap":349852}

{"status":"OK","reset":true}

Pytest Evidence:

_______________________________________________________________ test_reset_restores_safe_state _______________________________________________________________

device = <device.Device object at 0x0000023D9BDC0E90>

def test_reset_restores_safe_state(device):

device.command("SET LED ON")

device.command("RESET")

status = device.command("STATUS")

>       assert status["pwm"] == 0

E       assert 255 == 0

tests\test_firmware_cli.py:28: AssertionError

Severity: Critical

Reproducibility: 100% (відтворено 4 з 4 разів)

# Bug Report T-12

Title: Команда SET PWM 100 повернула AssertionError: assert 'ERROR' == 'OK'

Requirement: `SET PWM N` accepts every integer from 0 through 100 inclusive

Environment: Python 3.15.2, pytest 9.2.0, pyserial 3.5, DEVICE_PORT rfc2217://localhost:4000, Firmware BUGGY, Serial 115200

Preconditions: система після команди RESET {"status":"OK","reset":true}

Test/Steps: запустити тест test_pwm_valid_boundaries

Expected: test passed, {"status":"OK","pwm":100}

Actual: AssertionError: assert 'ERROR' == 'OK'

Raw Serial Evidence:

{"status":"OK","reset":true}

{"status":"ERROR","code":"INVALID_VALUE"}

{"status":"OK","reset":true}

Pytest Evidence:

_______________________________________________________________ test_pwm_valid_boundaries[100] _______________________________________________________________

device = <device.Device object at 0x0000023D9BCDA990>, value = 100

@pytest.mark.parametrize("value", [0, 1, 50, 99, 100])

def test_pwm_valid_boundaries(device, value):

r = device.command(f"SET PWM {value}")

>       assert r["status"] == "OK"

E       AssertionError: assert 'ERROR' == 'OK'

E

E         - OK

E         + ERROR

tests\test_firmware_cli.py:43: AssertionError

Severity: Major

Reproducibility: 100% (відтворено 4 з 4 разів)