UART / CLI Test Sheet

Webinar 3 — execution log + bug report

Environment

| Field | Value |
| --- | --- |
| Board | ESP32 / Wokwi build |
| Serial | 115200 |
| Build | Practice Buggy / Homework Buggy |
| Date | 06.10.2026 |

Execution Log

| ID | Input / Sequence | Pre-state | Expected | Actual | status after? | P/F |
| --- | --- | --- | --- | --- | --- | --- |
| TC-01 | help | Type 'help' | List of possible commands | Valid response – > Commands: help status set speed <0..255> set timeout <1..60> led on led off save reset > |  | P |
| TC-02 | status | Type command | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | Valid response- > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-03 | set speed 0 | SPEED=120 | > OK: speed=0 | Valid response - > OK: speed=0 | > SPEED=0 TIMEOUT=10 LED=OFF CONFIG=DIRTY | P |
| TC-04 | set speed 255 | SPEED=0 | > OK: speed=255 | Valid response – > OK: speed=255 | > SPEED=255 TIMEOUT=10 LED=OFF CONFIG=DIRTY | P |
| TC-05 | set speed 1 | SPEED=255 | > OK: speed=1 | Valid response – > OK: speed=1 | > SPEED=1 TIMEOUT=10 LED=OFF CONFIG=DIRTY | P |
| TC-06 | set speed 254 | SPEED=1 | > OK: speed=254 | Valid response – > OK: speed=254 | > SPEED=254 TIMEOUT=10 LED=OFF CONFIG=DIRTY | P |
| TC-07 | set speed -1 | SPEED=254 | Error response, speed must not be changed | Valid response – > ERROR: speed out of range | > SPEED=254 TIMEOUT=10 LED=OFF CONFIG=DIRTY | P |
| TC-08 | set speed 256 | SPEED=254 | Error response, speed must not be changed | Invalid response – > OK: speed=255 | > SPEED=255 TIMEOUT=10 LED=OFF CONFIG=DIRTY | F |
| TC-09 | set speed 122.4 | SPEED=255 | Error response, speed must not be changed | Valid response – > ERROR: invalid speed | > SPEED=255 TIMEOUT=10 LED=OFF CONFIG=DIRTY | P |
| TC-10 | set timeout 1 | TIMEOUT=10 | > OK: timeout=1 | Valid response - > OK: timeout=1 | > SPEED=255 TIMEOUT=1 LED=OFF CONFIG=DIRTY | P |
| TC-11 | set timeout 60 | TIMEOUT=1 | > OK: timeout=60 | Valid response - > OK: timeout=60 | > SPEED=255 TIMEOUT=60 LED=OFF CONFIG=DIRTY | P |
| TC-12 | set timeout 2 | After reset SPEED=120 TIMEOUT=60 LED=OFF CONFIG=SAVED | > OK: timeout=2 | Valid response – > OK: timeout=2 | > SPEED=120 TIMEOUT=2 LED=OFF CONFIG=DIRTY | P |
| TC-13 | set timeout 59 | TIMEOUT=2 | > OK: timeout=59 | Valid response – > OK: timeout=59 | > SPEED=120 TIMEOUT=59 LED=OFF CONFIG=DIRTY | P |
| TC-14 | set timeout 0 | TIMEOUT=59 | Error response, timeout must not be modified | Invalid response – > OK: timeout=0 | > SPEED=255 TIMEOUT=0 LED=OFF CONFIG=DIRTY | F |
| TC-15 | set timeout 61 | After reset TIMEOUT=10 | Error response, timeout must not be modified | Valid response – > ERROR: timeout out of range | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-16 | set timeout 32.7 | TIMEOUT=10 | Error response, timeout must not be modified | Valid response – > ERROR: invalid timeout | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-17 | RESET | Type 'help' | Unknown or incomplete commands must return a controlled ERROR | Valid response- > ERROR: unknown command | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-18 | STATUS | Type 'help' | Unknown or incomplete commands must return a controlled ERROR | Invalid response – > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | F |
| TC-19 | &&&%%% | Type 'help' | Special-character-only input must not be treated as a valid command | Invalid response – > OK | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | F |
| TC-20 | 11111 22222 33333 44444 55555 66666 77777 88888 – 40 characters | Type 'help' | Error message, device configuration must not be changed | Valid response – > ERROR: unknown command | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-21 | 88888 77777 66666 55555 44444 33333 22222 1111 – 39 characters | Type 'help' | Error message, device configuration must not be changed | Valid response – > ERROR: unknown command | > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-22 | 12345 67890 54321 09876 11111 22222 33333 44444 0 – 41 characters | Type 'help' | Error message, device configuration must not be changed | Invalid response – *** SYSTEM FAULT *** Reason: input line boundary failure rst:0xc (SW_CPU_RESET) Rebooting... ====================================== UART CLI HOMEWORK DEVICE v2.0 READY ====================================== Type 'help' | System crashed, rebooted | F |
| TC-23 | set | SPEED=200 TIMEOUT=10 LED=OFF CONFIG=DIRTY | Unknown or incomplete commands must return a controlled ERROR | > *** SYSTEM FAULT *** Reason: parser state mismatch while configuration is dirty rst:0xc (SW_CPU_RESET) Rebooting... ====================================== UART CLI HOMEWORK DEVICE v2.0 READY ====================================== | System crashed, rebooted | F |
| TC-24 | led on | SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | > OK status LED lights up with green | Valid response – > OK status LED lights up with green | > SPEED=120 TIMEOUT=10 LED=ON CONFIG=SAVED | P |
| TC-25 | led off | SPEED=120 TIMEOUT=10 LED=ON CONFIG=SAVED | > OK status LED turns off | Valid response – > OK status LED turns off | SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-26 | reset | SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | > OK: reset | Valid response – > OK: reset | SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-27 | save | Set speed 100, SPEED=100 TIMEOUT=10 LED=OFF CONFIG=DIRTY | > OK: configuration saved | Valid response – > OK: configuration saved | SPEED=100 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-28 | empty row | SPEED=100 TIMEOUT=10 LED=OFF CONFIG=SAVED | Unknown or incomplete commands must return a controlled ERROR | Valid response – > ERROR: empty command | SPEED=100 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |
| TC-29 | Type space symbol | SPEED=100 TIMEOUT=10 LED=OFF CONFIG=SAVED | Unknown or incomplete commands must return a controlled ERROR | Valid response – > ERROR: empty command | SPEED=100 TIMEOUT=10 LED=OFF CONFIG=SAVED | P |

Bug Report for TC-08

Title: Встановлення значення speed = 256 (max+1) приймається системою

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: Система у робочому стані, значення SPEED=254

Steps to Reproduce: Увести команду у консолі set speed 256

Expected: При введенні значення для параметра speed поза межами 0..255 система має повернути помилку і значення speed не змінюється

Actual: Система повертає невалідну відповідь > OK: speed=255

Evidence:

System LOG:

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK: speed=255

> SPEED=255

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

> OK: speed=255

> SPEED=255

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

> OK: speed=255

> SPEED=255

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

> OK: speed=255

> SPEED=255

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

> OK: speed=255

> SPEED=255

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

Severity: High

Reproducibility: 100% (відтворено 5 з 5 разів)

Bug Report for TC-14

Title: Встановлення значення timeout = 0 (min-1) приймається системою

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: Система у робочому стані, значення TIMEOUT=60

Steps to Reproduce: Увести команду у консолі set timeout 0

Expected: При введенні значення для параметра timeout поза межами 1..60 система має повернути помилку і значення timeout не змінюється

Actual: Система повертає невалідну відповідь > OK: timeout=0, значення timeout змінилось на 0

Evidence:

System LOG:

> SPEED=120

TIMEOUT=60

LED=OFF

CONFIG=DIRTY

> OK: timeout=0

> SPEED=120

TIMEOUT=0

LED=OFF

CONFIG=DIRTY

> OK: timeout=0

> SPEED=120

TIMEOUT=0

LED=OFF

CONFIG=DIRTY

> OK: timeout=0

> SPEED=120

TIMEOUT=0

LED=OFF

CONFIG=DIRTY

> OK: timeout=0

> SPEED=120

TIMEOUT=0

LED=OFF

CONFIG=DIRTY

> OK: timeout=0

> SPEED=120

TIMEOUT=0

LED=OFF

CONFIG=DIRTY

Severity: High

Reproducibility: 100% (відтворено 5 з 5 разів)

Bug Report for TC-18

Title: Система валідно відповідає на команду STATUS

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: Система у робочому стані

Steps to Reproduce: Увести команду у консолі STATUS

Expected: Відносно файлу Requirements.md команди є case-sensitive, тому невалідна команда не має змінювати конфігурацію пристрою, не має сприйматись системою

Actual: Система повертає відповідь > SPEED=120 TIMEOUT=10 LED=OFF CONFIG=SAVED, хоча мала повернути ERROR: unknown command як у TC-17

Evidence:

System LOG:

type STATUS

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

type STATUS

type STATUS

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

type STATUS

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

type STATUS

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

type STATUS

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

Severity: Major

Reproducibility: 100% (відтворено 5 з 5 разів)

Bug Report for TC-19

Title: Введення спец. символів &&&%%% у вигляді команди сприймається системою як валідна команда

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: Система у робочому стані

Steps to Reproduce: Увести команду у консолі &&&%%%

Expected: Відносно файлу Requirements.md введення спец. символів у вигляді команди не повинно сприйматись системою як валідна команда

Actual: Система повертає відповідь > OK, хоча мала повернути ERROR: unknown command

Evidence:

System LOG:

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

Severity: High

Reproducibility: 100% (відтворено 5 з 5 разів)

Bug Report for TC-22

Title: Введення 41 символа (max+1) у рядку консолі призводить до краша та перезавантаження системи

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: Система у робочому стані

Steps to Reproduce: Увести команду у консолі 12345678905432109876111112222233333444440

Expected: Відносно файлу Requirements.md на будь-яку команду довше 40 символів система повинна відповісти помилкою ‘Error: command too long’ і це не має призвести до перезавантаження, зависанню або пошкодженню стану парсера

Actual: Система повертає

>

*** SYSTEM FAULT ***

Reason: input line boundary failure

rst:0xc (SW_CPU_RESET)

Rebooting...

Evidence:

System LOG:

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

>

*** SYSTEM FAULT ***

Reason: input line boundary failure

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

>

*** SYSTEM FAULT ***

Reason: input line boundary failure

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

>

*** SYSTEM FAULT ***

Reason: input line boundary failure

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

>

*** SYSTEM FAULT ***

Reason: input line boundary failure

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

>

*** SYSTEM FAULT ***

Reason: input line boundary failure

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

>

Severity: High

Reproducibility: 100% (відтворено 5 з 5 разів)

Bug Report for TC-23

Title: Введення неповної команди (set) у рядку консолі  коли CONFIG=DIRTY призводить до краша та перезавантаження системи

Environment: ESP32 DevKit v1, Wokwi Simulator, Serial Monitor (115200 baud), Build: BUGGY

Preconditions: Система у робочому стані

Steps to Reproduce: 1) Ввести команду у консолі set speed 200; 2) Ввести команду status; 3) Ввести команду set

Expected: Система має відповісти помилкою (наприклад Error: invalid command), система має бути стабільною, не перезавантажуватись чи крашитись

Actual: Система повертає

>

*** SYSTEM FAULT ***

Reason: parser state mismatch while configuration is dirty

rst:0xc (SW_CPU_RESET)

Rebooting...

Evidence:

System LOG:

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK: speed=200

> SPEED=200

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

>

*** SYSTEM FAULT ***

Reason: parser state mismatch while configuration is dirty

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK: speed=150

> SPEED=150

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

>

*** SYSTEM FAULT ***

Reason: parser state mismatch while configuration is dirty

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK: speed=180

> SPEED=180

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

>

*** SYSTEM FAULT ***

Reason: parser state mismatch while configuration is dirty

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> ERROR: unknown command

> OK: speed=250

> SPEED=250

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

>

*** SYSTEM FAULT ***

Reason: parser state mismatch while configuration is dirty

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

> SPEED=120

TIMEOUT=10

LED=OFF

CONFIG=SAVED

> OK: speed=200

> SPEED=200

TIMEOUT=10

LED=OFF

CONFIG=DIRTY

>

*** SYSTEM FAULT ***

Reason: parser state mismatch while configuration is dirty

rst:0xc (SW_CPU_RESET)

Rebooting...

======================================

UART CLI HOMEWORK DEVICE v2.0

READY

======================================

Type 'help'

>

Severity: High

Reproducibility: 100% (відтворено 5 з 5 разів)

QA Conclusion

CLI підтримує основні happy-path команди, але robustness не відповідає вимогам. Boundary testing виявляє некоректну обробку команд speed=256 та timeout=0. Parsing test показує порушення case-sensitive requirements для команди STATUS і помилкову валідну відповідь системи на введення special-character-only input. Input-length testing виявляє unexpected reset системи при введенні 41 символу. State/sequence testing також знаходить дефект, який проявляється лише у DIRTY state: incomplete command ‘set’ може призвести до крашу і перезавантаження пристрою. Після negative tests необхідно перевіряти status, оскільки controlled ERROR має зберігати доступність CLI та state.Build не можна вважати release-ready до виправлення reset/crash paths та validation defects.